# Threat Prioritization Plan

# Revision

Version 15

7/9/26 11:17 AM

# SME

Charles Wilson

# Abstract

This document details the process to be used to translate the threat candidates identified by various cybersecurity activities into actionable issues able to be dispositioned by development teams.

# Group / Owner

Security / Systems Requirements Planner

# Motivation

This document is motivated by the need to have formal processes in place regarding the disposition of cybersecurity-related threats in the context of the development of safety-critical, cyber-physical systems.

Note:	Within the context of this document, the terms security and cybersecurity are used interchangeably. It is presumed that the term security is being used in reference to cybersecurity and not physical security.

# License

This work was originally created by Motional and is licensed under the Creative Commons Attribution-Share Alike (CC BY-SA-4.0) License.

https://creativecommons.org/licenses/by/4.0/legalcode

# Overview

The following diagram illustrates the process to be used:

Note:	The above methodology follows that described in guidance from NIST SP 800-30 [9].

# Process

## Threat Candidate Ranking

| Inputs | Triaged threat candidates |
| --- | --- |
| Outputs | Ranked threat candidates |
| Participants | Security SME |

The triaged threat candidates are ranked in order to establish their relative exploitability. This should be done using a standard ranking methodology such as the Common Vulnerability Scoring System (CVSS) [5].

The ranking should consider aspects such as:

- Mechanism

- Locality

- Maturity

- Scope

- Required privileges

The ranking system may lend itself to coarse ranking (by range).

Once ranked, a threshold may be applied to reduce the number of threat candidates needing to be considered in the risk step. If a threshold is applied, the dismissed candidates must be documented in the candidate’s source tracker.

Note:	The triaged threat candidates may be sourced from several activities, including:

- Threat Modeling [2]

- Incident Response [3]

- Attack Surface Analysis [4]

- Deprecated Functions [12]

- Static Analysis [13]

- Dynamic Analysis [14]

- Fuzz Testing [15]

- Penetration Testing [16]

## Ranked Threat Candidate Risking

| Inputs | Ranked threat candidates |
| --- | --- |
| Outputs | Risked threat candidates Ranked / risked threat report |
| Participants | Risk SME |

The ranked threat candidates meeting the threshold of consideration are reviewed by the risk SME to determine their severity of harm. A ranked / risked threat report is generated.

## Threat Candidate Slicing

| Inputs | Risked threat candidates |
| --- | --- |
| Outputs | Uncontrolled threats Threat report |
| Participants | Security SME |

The threat candidates which have been both ranked and risked are sliced (bisected) to determine which ones are considered to have uncontrolled risk. These are designated as uncontrolled threats and forwarded to the issue tracking system. If an issue is determined to be controlled, an update dismissed candidate notification is sent to the source tracker to document this. A threat report is generated.

Note:	It is the responsibility of the standard issue management processes to dispose of uncontrolled issues applying the standard treatments (avoid, reduce, share, retain). This includes documenting the risk aspects of the issue and the reason for the disposition.

The following diagram illustrates issue slicing.

Note:	Adapted from Postmarket Management of Cybersecurity in Medical Devices [10].

Here we are showing the values from ranking (likelihood) and risking (impact) as continuous. The level of risk control is also shown as continuous from white (completely controlled) to black (completely uncontrolled). In practice, all of these values are quantized. Finally, our desire is to bisect the space by choosing a minimum level of acceptable risk. This yields a line (risk appetite) through the exploitability-severity space above which threats are deemed to be uncontrolled (in need of mitigation), as indicated with the red line and arrow.

Note:	Additional information regarding the mapping of various ranking and risking methodologies into the notation specified by ISO 21434 can be found in the Ranked / Risked Threat Report [7].

Note:	The issue of reporting when multiple risk dimensions are considered, as by ISO 21434, is addressed in the Threat Report [8].

If a threat candidate is determined to be controlled, it is dismissed and must be documented in the candidate’s source tracker. Uncontrolled threats will be entered into the defect tracking system for disposition by development.

Note:	The bisection line is determined by the organization’s risk acceptance level.

# Multi-risk Domain Considerations

In those situations where multiple risk domains need to be taken into consideration, such as those specified in EVITA [11], the following diagram illustrates the process to be used:

Here we see that in addition to the risk and slicing processes being replicated for each of the risk domain, the output of these must be consolidated and the threat de-duplicated.

## Consolidate Risk Domains

| Inputs | Uncontrolled threats (all domains) |
| --- | --- |
| Outputs | Consolidated uncontrolled threats Consolidated threat report |
| Participants | Security SME |

The uncontrolled threats from all risk domains are consolidated by the security SME. This results in the creation of a set of consolidated uncontrolled threats and a consolidated threat report. The consolidated uncontrolled threats are entered into the issue tracking system.

Note:	The consolidated uncontrolled threats should be deduplicated.

# References

- Document Management Standard

https://en.wikipedia.org/wiki/Document_management_system

- Threat Modeling Report (AVCDL secondary document)

- Incident Response Plan (AVCDL secondary document)

- Attack Surface Analysis Report (AVCDL secondary document)

- Common Vulnerability Scoring System
https://www.first.org/cvss/

- NIST SP 800-39 - Managing Information Security Risk - Organization, Mission, and Information System View
https://nvlpubs.nist.gov/nistpubs/Legacy/SP/nistspecialpublication800-39.pdf

- Ranked / Risked Threat Report (AVCDL secondary document)

- Threat Report (AVCDL secondary document)

- NIST SP 800-30 r1 - Guide for Conducting Risk Assessments
https://nvlpubs.nist.gov/nistpubs/Legacy/SP/nistspecialpublication800-30r1.pdf

- Postmarket Management of Cybersecurity in Medical Devices
https://www.fda.gov/regulatory-information/search-fda-guidance-documents/postmarket-management-cybersecurity-medical-devices

- EVITA D2.3 Security requirements for automotive on-board networks based on dark-side scenarios
https://zenodo.org/record/1188418/files/EVITAD2.3v1.1.pdf

- Currently Used Deprecated Functions Document (AVCDL secondary document)

- Static Analysis Report (AVCDL secondary document)

- Dynamic Analysis Report (AVCDL secondary document)

- Fuzz Testing Report (AVCDL secondary document)

- Penetration Testing Report (AVCDL secondary document)